Task 1: See Appendix B; Task 2 Plot: See below

Task 3 Plot: R=4.9042m, σ=0

Task 4 Results:

- Delivered current: IL = 125 A

- Vcell = 1.48V (recharge, Vin); VL = 0.9765V (discharge, Vout); Vin/Vout = 0.658.

Appendix A: Matlab Code

Task2.m

rau = 1.18
v1 = 12
alpha = 8
CL = 1.27
n = 3
K_h = 2.7
const = 0.4
r_h = 2
R = 35
w = 2
lamda = w*R/v1
r_head = r_h/R
x = const/(1-r_head)
power1 = (1/3)*n*rau*v1^3*CL*lamda^2*K_h*R
a = (((1/3)+(const/(1-r_head))*((r_head/3)-1/4))*(1+(4/(9*lamda^2)))^1.5)-(((1/3)+(const/(1-r_head))*((r_head/12)))*(r_head+(4/(9*lamda^2)))^1.5)
b = x*1/(18*lamda^2)*((sqrt(1+(4/(9*lamda^2)))-(r_head*sqrt(r_head^2+(4/(9*lamda^2))))))
c = x*2/(81*lamda^4)*log((1+sqrt(1+(4/(9*lamda^2))))/(r_head+sqrt(r_head^2+(4/(9*lamda^2)))))
ans = power1*(a-b-c)
Wbetz = 8*pi/27*rau*v1^3*R^2*(1-r_head^2);

eff = (9*n*CL*lamda^2*K_h)*(a-b-c)/(8*pi*R*(1-r_head^2))

Task2Plot.m

%% Task 2 Problem Statement

%% Constants/Initializations

p = 1.18; %[kg/m3]
v1 = 12; %[m/s]
a = 8; %[deg]
CL = 1.27; %unitless, coeff. of lift
n = 3; %unitless, 3 blades
Kh = 2.7; %[m], chord at hub r = rh
o = 0.40; %unitless, taper ratio
rh = 2.0; %[m], radius at hub
R = 35.0; %[m], radius at tip
w = 2.0; %[rad/s], angular rotation speed

lambda = w*R/v1;
rhat = rh/R;

%% Plot

r = linspace(rh,R,100); %[m], linspace
w_deg = rad2deg(w);
zeta = a - atand(2*R*v1./(3*w_deg*R.*r)); %[deg]
plot(r,zeta);
xlabel("Rotor Radius [m]");
ylabel("Setup Angle \xi [deg.]");
title("Task 2 Plot: Setup Angle vs. Radius for \alpha = 8 deg.");

Task3.m

%% Task 3 Problem Statement
% Provide 1.5 kW of power. This means Wdot = 1.5e3.
% Maximize efficiency, Wdot/Wbetz

p = 1.18;
v1 = 6.5;
alpha = 8;
CL = 1.26;
n = 3;
w = 3.0;

Wdot_des = 1500;
ospace = linspace(0,1,101); % Deliverable
Rspace = linspace(2,190,17001); % Deliverable

% power_results = zeros(31,11);
% eff_results = zeros(31,11);
results = [];
for i = 1:length(ospace)
    for j = 1:length(Rspace)
        o = ospace(i);
        R = 4.904;%Rspace(j);
        rh = 0.1*R;
        Kh = 0.085*R;
        lambda = w*R/v1;
        rhat = rh/R;

        x = o/(1-rhat);
        power1 = (1/3)*n*p*v1^3*CL*lambda^2*Kh*R;
        a = (((1/3)+(o/(1-rhat))*((rhat/3)-1/4))*(1+(4/(9*lambda^2)))^1.5)-(((1/3)+(o/(1-rhat))*((rhat/12)))*(rhat+(4/(9*lambda^2)))^1.5);
        b = x*1/(18*lambda^2)*((sqrt(1+(4/(9*lambda^2)))-(rhat*sqrt(rhat^2+(4/(9*lambda^2))))));
        c = x*2/(81*lambda^4)*log((1+sqrt(1+(4/(9*lambda^2))))/(rhat+sqrt(rhat^2+(4/(9*lambda^2)))));
        Wdot = power1*(a-b-c);
        Wbetz = 8*pi/27*p*v1^3*R^2*(1-rhat^2);

        eff = (9*n*CL*lambda^2*Kh)*(a-b-c)/(8*pi*R*(1-rhat^2));
        
        error = abs(Wdot-Wdot_des);
        if(error < 0.025*Wdot_des) && (eff<1)% && (eff>0.3)
            col = [o; R; Wdot; eff];
            results = [results col];
        end
        power_results(i,j) = Wdot;
        eff_results(i,j) = eff;
    end
end

power_results = [Rspace; power_results];
eff_results = [Rspace; eff_results];

Task3Plot.m

%% Task 3 Problem Statement
% Plot setup angle to keep alpha = 8

%% Constants/Initializations

p = 1.18;
v1 = 6.5;
a = 8;
CL = 1.26;
n = 3;
w = 3.0;

% Deliverables
o = 0;
R = 4.9042; %[m]

rh = 0.1*R;
Kh = 0.085*R;
lambda = w*R/v1;
rhat = rh/R;

%% Plot

r = linspace(rh,R,100); %[m], linspace
w_deg = rad2deg(w);
zeta = a - atand(2*R*v1./(3*w_deg*R.*r)); %[deg]
plot(r,zeta);
xlabel("Rotor Radius [m]");
ylabel("Setup Angle \xi [deg.]");
title("Task 3 Plot: Setup Angle vs. Radius for \alpha = 8 deg.");

Appendix B: Task 1 Derivation